ASSIGNMENT 2 FRONT SHEET

| Qualification | TEC Level 5 HND Diploma in Computing |  |  |
| --- | --- | --- | --- |
| Unit number and title | Unit 2: Networking |  |  |
| Submission date |  | Date Received 1st submission |  |
| Re-submission Date |  | Date Received 2nd submission |  |
| Student Name | Pham Xuan Hai | Student ID | BH00132 |
| Class | IT0501 | Assessor name | Nguyen Hong hanh |
| Student declaration I certify that the assignment submission is entirely my own work and I fully understand the consequences of plagiarism. I understand that making a false declaration is a form of malpractice. |  |  |  |
|  |  | Student’s signature | hai |

Grading grid

| P5 | P6 | P7 | P8 | M3 | M4 | D2 |
| --- | --- | --- | --- | --- | --- | --- |

| Summative Feedback: Resubmission Feedback: |  |  |
| --- | --- | --- |
| Grade: | Assessor Signature: | Date: |
| Internal Verifier’s Comments: |  |  |
| Signature & Date: |  |  |

Table of Contents

A.	Introduction	5

B.	Content	5

P5. Design a networked system to meet a given specification.	5

1.	The difference between logical and physical design	5

2.	The USER Requirement for the design	5

3.	Logical design of the network based on user requirement	6

4.	Physical design of the network based on user requirement	6

5.	Addressing table for the network you design	7

P6. Design a maintenance schedule to support the networked system.	8

1.	Network maintenance definition.	8

2.	Task for the maintenance plan.	8

3.	Maintenance schedule	9

P7. Implement a networked system based on a prepared design.	10

P8. Document and analyze test results against expected results	17

‌2. Testting.	23

C.	Conclusion	30

D. Reference	30

Figure 1:Logical design	6

Figure 2:Physical design	7

Figure 3:Cisco Package Tracer Instructor to design an emulator for my network.	10

Figure 4:Set passwords for console and enable mode.	10

Figure 5: Set the domain, mode, and VTP password.	11

Figure 6:Configure trunk ports on interfaces f0/1-4	11

Figure 7:Sw-Staff	12

Figure 8:Sw-Student	12

Figure 9:IP address for each VLAN	12

Figure 10:relay Configuration	13

Figure 11:Assign VLANs to ports	13

Figure 12:Routing Configuration	13

Figure 13:Border Router Configuration	14

Figure 14:ISP Router Configuration	15

Figure 15:Internet Router Configuration	15

Figure 16:Servers DHCP configuration	16

Figure 17:Boder router configuration	17

Figure 20:Check password	23

Figure 21:Check configure VTP and check status	23

Figure 22:Check Vlan	23

Figure 23:Check Trunking	24

Figure 24:Check configure VTP and check status of Sw- server	24

Figure 25:Check configure VTP and check status of Sw- staff	25

Figure 26:Check configure VTP and check status of Sw- student	25

Figure 27:Check services DHCP	26

Figure 28:TC-1 computer is dynamically assigned IP by DHCP	27

Figure 29:Ping Realtime	27

Figure 30:Check ping after configuring an access control list	28

Figure 31:Check ping from outside to inside after configuring NAT	28

# Introduction

Nguyen Networking Limited is embarking on a pivotal networking endeavor tailored for a local educational institution. This institution boasts a dynamic community, comprising 200 students, 15 teachers, and 12 staff members, all housed within a multi-floor facility. In addition to its human resources, the institution features specialized assets such as student lab computers and printers, strategically positioned throughout the premises.

The overarching goal of this project is to optimize connectivity and collaboration within the institution, with a keen focus on three key objectives: ensuring seamless connectivity, implementing robust security measures, and facilitating streamlined access to resources across all levels of the organization. Nguyen Networking Limited is committed to leveraging its expertise and state-of-the-art solutions to construct a network infrastructure that not only meets but exceeds these objectives.

Through a combination of meticulous planning, efficient deployment strategies, and unwavering ongoing support, we are poised to revolutionize the institution's technological landscape. Our aim is to create an environment that fosters innovation, facilitates seamless collaboration, and empowers educational excellence. With Nguyen Networking Limited at the helm, the institution's stakeholders will be equipped to navigate and thrive in today's rapidly evolving digital landscape.

# Content

## P5. Design a networked system to meet a given specification.

### The difference between logical and physical design

| Physical Design | Logical Design |
| --- | --- |
| The physical design is highly detailed. | Logical design is a high-level design and doesn’t provide any detail. |
| Physical design is more graphical than textual; however, it can comprise both. | Logical design can be textual, graphic, or both. |
| A physical design focuses on specific solutions explaining how they are assembled or configured | A logical design focuses on satisfying the design factors, including risks, requirements, constraints, and assumptions. |

### The USER Requirement for the design

User requirements for the design:

- Building: 3 floors, all computers and printers are on the ground floor apart from the IT labs – one lab located on the first floor and another located on the second floor.

- People: 200 students, 15 teachers, 12 marketing and administration staff, 5 higher managers including the head of academics and the program manager, and 3 computer network administrators.

- Resources: 50 student lab computers, 35 staff computers, 3 printers.

- When implementing, ensure stability, clear hierarchy, simplicity, and affordability.

### Logical design of the network based on user requirement

Figure 1:Logical design

### Physical design of the network based on user requirement

Figure 2:Physical design

### Addressing table for the network you design

| Type of user | Vlan | Network Address | Subnets Mask | Default Gateway | DHCP | DNS |
| --- | --- | --- | --- | --- | --- | --- |
| Teacher | 17 | 192.168.17.0 | 255.255.255.224 | 192.168.17.1 | 192.168.107.2 | 192.168.107.3 |
| SEO | 27 | 192.168.27.0 | 255.255.255.240 | 192.168.27.1 | 192.168.107.2 | 192.168.107.3 |
| Head | 37 | 192.168.37.0 | 255.255.255.248 | 192.168.37.1 | 192.168.107.2 | 192.168.107.3 |
| IT | 47 | 192.168.47.0 | 255.255.255.248 | 192.168.47.1 | 192.168.107.2 | 192.168.107.3 |
| Student | 57 | 192.168.57.0 | 255.255.255.0 | 192.168.57.1 | 192.168.107.2 | 192.168.107.3 |
| server | 107 | 192.168.107.0 | 255.255.255.248 | 192.168.107.1 | 192.168.107.2 | 192.168.107.3 |

Explain the subnetmask division:

Split Subnet Mask brings some benefits as follows:

Easier network management: Dividing the network into smaller subnets makes it easier to manage devices on the network.

Enhance network security: Split subnet mask helps isolate subnets from each other, helping to increase network security.

Optimal performance network: Split network mask helps minimize minimum broadcast, helping to optimize network performance.

## P6. Design a maintenance schedule to support the networked system.

### Network maintenance definition.

Network maintenance is essentially what you need to do to keep your network up and running smoothly. This definition encompasses some duties such as installing and configuring hardware and software, troubleshooting network problems, monitoring and improving network performance and planning for network growth.

### Task for the maintenance plan.

- Troubleshooting Problems: Proactively identify and resolve network issues, distinguishing between internal and external causes. Utilize network monitoring tools for early detection.

- Performing Data and Configuration Backups: Regularly back up critical data and network configurations, ensuring backups are accessible, verified, and up to date.

- Device Inventory Management: Maintain an accurate inventory of all network devices, tracking their status, updates, and lifecycle for efficient maintenance and replacement.

- Malware/Ransomware Protection: Implement robust security measures to protect against evolving threats, including regular updates, scans, and configuration adjustments.

- Power Checks: Test and maintain UPS systems to ensure uninterrupted power supply, scaling capacity with network growth.

- Network Documentation: Document all network components, maintenance activities, and relevant details to facilitate troubleshooting and knowledge transfer.

- Hardware Checks: Regularly inspect and maintain network hardware for damage, dust, and connectivity issues to prevent failures.

- Compliance Checks: Ensure network operations comply with legal and industry regulations, implementing necessary policies and security standards.

- Preemptive Repairs: Conduct scheduled maintenance to address potential issues before they disrupt network operations.

- Configurations and Upgrades: Regularly update software and devices to address security vulnerabilities and accommodate organizational changes.

- Future Network Growth Planning: Plan for network scalability and expansion based on past performance, recurring issues, and anticipated needs.

### Maintenance schedule

|  | Daily | Weekly | Monthly | Yearly |
| --- | --- | --- | --- | --- |
| System Checks: Check for any physical damage or signs of wear on networking equipment. | x |  |  |  |
| Software Updates |  | x |  |  |
| Cable and Physical Infrastructure Inspection |  |  | x |  |
| Data Backup | x |  |  |  |
| User Account Management. |  | x |  |  |
| Monitoring Network Performance: |  |  | x |  |
| Server access reviewed |  | x |  |  |
| Firewall Rules reviewed |  | x |  |  |
| Hardware physically cleaned and errors checked |  |  | x |  |
| System Performance Optimization |  |  | x |  |
| Security Checks |  | x |  |  |
| System Error Checking and Handling |  |  | x |  |
| Security Audits |  |  |  | x |
| Check and Replace Old Hardware |  |  |  | x |
| Scaling and Future Planning |  |  |  | x |

## P7. Implement a networked system based on a prepared design.

Figure 3:Cisco Package Tracer Instructor to design an emulator for my network.

Here are the specific steps to configure each part of the network:

- Sw Core:

- Security Configuration:

- Set passwords for console and enable mode.

Figure 4:Set passwords for console and enable mode.

- VTP Configuration:

- Set the domain, mode, and VTP password.

Figure 6:Name the Vlans

Figure 5: Set the domain, mode, and VTP password.

Name the Vlans:

- Trunking Configuration:

Figure 6:Configure trunk ports on interfaces f0/1-4

Configure trunk ports on interfaces f0/1-4.

- Sw Server

- Sw-Staff:

Figure 7:Sw-Staff

Figure 8:Sw-Student

Sw-Student:

Figure 9:IP address for each VLAN

IP address for each VLAN

- relay Configuration:

Figure 10:relay Configuration

- Assign VLANs to ports

Figure 11:Assign VLANs to ports

- Routing Configuration

Figure 12:Routing Configuration

- Border Router Configuration

Figure 13:Border Router Configuration

- ISP Router Configuration:

Figure 14:ISP Router Configuration

- Internet Router Configuration

Figure 15:Internet Router Configuration

- Servers DHCP configuration

Figure 16:Servers DHCP configuration

- Boder router configuration:

Figure 17:Boder router configuration

### P8. Document and analyze test results against expected results

- Set passwords for console and enable mode

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SW-core

SW-core(config)#line console 0

SW-core(config-line)#password 123

SW-core(config-line)#login

Switch>en

Switch#conf t

Switch(config)#Enable secret admin

Switch(config)#exit

- Set the domain, mode, and VTP password.

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

SW-core(config)#line vty 0 4

SW-core(config-line)#password cissco123

SW-core(config-line)#login

SW-core(config-line)#^Z

SW-core#

%SYS-5-CONFIG_I: Configured from console by console

conf t

Enter configuration commands, one per line. End with CNTL/Z.

SW-core(config)#vtp domain btec.vn

Changing VTP domain name from NULL to btec.vn

SW-core(config)#vtp mode server

Device mode already VTP SERVER.

SW-core(config)#vtp password btec123

- Configure trunk ports on interfaces f0/1-4.

Setting device VLAN database password to btec123

SW-core(config)#int range f0/1-4

SW-core(config-if-range)#sw trunk enc dot1Q

SW-core(config-if-range)#sw mode trunk

SW-core(config-if-range)#end

- Name the Vlans

SW-core#en

SW-core#conf t

Enter configuration commands, one per line. End with CNTL/Z.

SW-core(config)#ip routing

SW-core(config)#vlan 17

SW-core(config-vlan)#name teacher

SW-core(config-vlan)#vlan 27

SW-core(config-vlan)#name SEO

SW-core(config-vlan)#vlan 37

SW-core(config-vlan)#name Head

SW-core(config-vlan)#vlan 47

SW-core(config-vlan)#name IT

SW-core(config-vlan)#vlan 57

SW-core(config-vlan)#name student

SW-core(config-vlan)#vlan 107

SW-core(config-vlan)#name server

SW-core(config-vlan)#int vlan 17

- IP address for each vlan

SW-core(config-vlan)#int vlan 17

SW-core(config-if)#ip add 192.168.17.1 255.255.255.224

SW-core(config-if)#no shutdown

SW-core(config-if)#int vlan 27

SW-core(config-if)#ip add 192.168.27.1 255.255.255.240

SW-core(config-if)#no shutdown

SW-core(config-if)#int vlan 37

SW-core(config-if)#ip add 192.168.37.1 255.255.255.248

SW-core(config-if)#no shutdown

SW-core(config-if)#int vlan 47

SW-core(config-if)#ip add 192.168.47.1 255.255.255.248

SW-core(config-if)#no shutdown

SW-core(config-if)#int vlan 57

SW-core(config-if)#ip add 192.168.57.1 255.255.255.0

SW-core(config-if)#no shutdown

SW-core(config-if)#int vlan 107

SW-core(config-if)#ip add 192.168.107.1 255.255.255.248

SW-core(config-if)#no shutdown

SW-core(config-if)#exit

- relay Configuration:

SW-core(config)#int vlan 17

SW-core(config-if)#ip helper-address 192.168.107.2

SW-core(config-if)#int vlan 27

SW-core(config-if)#ip helper-address 192.168.107.2

SW-core(config-if)#int vlan 37

SW-core(config-if)#ip helper-address 192.168.107.2

SW-core(config-if)#int vlan 47

SW-core(config-if)#ip helper-address 192.168.107.2

SW-core(config-if)#int vlan 57

SW-core(config-if)#ip helper-address 192.168.107.2

- Assign VLANs to ports

SW-core(config)#access-list 110 deny ip 192.168.17.0.0.0.0.255 192.168.27.0.0.0.0.31

SW-core(config)#access-list 110 deny ip 192.168.17.0.0.0.0.255 192.168.37.0.0.0.0.15

SW-core(config)#access-list 110 deny ip 192.168.17.0.0.0.0.255 192.168.47.0.0.0.0.7

- Routing Configuration

SW-core(config-if)#int g0/1

SW-core(config-if)#ip address 1.1.1.1 255.255.255.252

SW-core(config-if)#no shutdown

- Configuration on Sw-staff

Switch(config)#hostname SW-Staff

SW-Staff(config)#vtp domain btec.vn

Changing VTP domain name from NULL to btec.vn

SW-Staff(config)#vtp mode client

Setting device to VTP CLIENT mode.

SW-Staff(config)#vtp pass btec123

Setting device VLAN database password to btec123

SW-Staff(config)#int f0/1

SW-Staff(config-if)#sw mode trunk

SW-Staff(config-if)#int range f0/2-10

SW-Staff(config-if-range)#sw mode acc

SW-Staff(config-if-range)#sw acc vlan 17

SW-Staff(config-if-range)#int range f0/11-15

SW-Staff(config-if-range)#sw mode acc

SW-Staff(config-if-range)#sw acc vlan 27

SW-Staff(config-if-range)#int range f0/16-21

SW-Staff(config-if-range)#sw mode acc

SW-Staff(config-if-range)#sw acc vlan 37

SW-Staff(config-if-range)#int range f0/22-24

SW-Staff(config-if-range)#sw mode acc

SW-Staff(config-if-range)#sw acc vlan 47

- Configuration above Sw-server

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SW-server

SW-server(config)#vtp domain btec.vn

Domain name already set to btec.vn.

SW-server(config)#vtp mode client

Setting device to VTP CLIENT mode.

SW-server(config)#vtp pass btec123

Setting device VLAN database password to btec123

SW-server(config)#int f0/1

SW-server(config-if)#sw mode trunk

SW-server(config-if)#int range f0/2-4

SW-server(config-if-range)#sw mode acc

SW-server(config-if-range)#sw acc vlan 107

- ‌ Configuration on Sw-student

Sw- student 1

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SW-student1

SW-student1(config)#vtp domain btec.vn

Domain name already set to btec.vn.

SW-student1(config)#vtp mode client

Setting device to VTP CLIENT mode.

SW-student1(config)#vtp pass btec123

Setting device VLAN database password to btec123

SW-student1(config)#int f0/1

SW-student1(config-if)#sw mode trunk

SW-student1(config-if)#int range f0/2-10

SW-student1(config-if-range)#sw mode acc

SW-student1(config-if-range)#sw acc vlan 57

- Border Router Configuration:

Router(config)#ip route 192.168.0.0 255.255.0.0 10.10.12.1

Router(config)#ip route 0.0.0.0 0.0.0.0 1.1.1.2

Router(config)#int g0/0

Router(config)#ip add 1.1.1.2 255.255.255.252

Router(config)#no sh

- ISP Router Configuration:

Router(config)#int g0/0

Router(config-if)#no shut

Router(config-if)#ip add 2.2.2.1 255.255.255.252

Router(config-if)#ex

Router(config)# ip route 0.0.0.0 0.0.0.0 2.2.2.2

Router(config)#int g0/1

Router(config-if)#no shut

Router(config-if)#ex

Router(config)# ip route 0.0.0.0 0.0.0.0 3.3.3.2

### ‌2. Testting.

- Testing.

Figure 18:Check password

Check password

- Check configure VTP and check status.

Figure 19:Check configure VTP and check status

Figure 20:Check Vlan

Check Vlan:

Check Trunking

Figure 21:Check Trunking

Figure 22:Check configure VTP and check status of Sw- server

Check configure VTP and check status of Sw- server:

- Check configure VTP and check status of Sw- staff:

Figure 23:Check configure VTP and check status of Sw- staff

- Check configure VTP and check status of Sw- student:

Figure 24:Check configure VTP and check status of Sw- student

- Check services DHCP:

Figure 25:Check services DHCP

- TC-1 computer is dynamically assigned IP by DHCP:

Figure 26:TC-1 computer is dynamically assigned IP by DHCP

- Ping Realtime:

Figure 27:Ping Realtime

Figure 31:Check computers with different VLANs can ping each other

Figure 28:Check ping after configuring an access control list

Check ping after configuring an access control list:

- Check cmputers with different VLANs can ping each other:

Figure 29:Check ping from outside to inside after configuring NAT

Check ping from outside to inside after configuring NAT:

Check DNS web serve:

# Conclusion

In conclusion, I have presented the required contents of the task are: Provide a logical and physical design of the networked system with clear explanation and addressing table; Evaluate the design to meet the requirements; Implement a networked system based on a prepared design; Document and analyze test results against expected results in this report.

# D. Reference

Business, A. (2022). Understanding the Logical and Physical Design of IoT. [online] Airtel B2B Blog. Available at: https://www.airtel.in/blog/business/physical-and-logical-design-of-iot/#:~:text=Logical%20design%20is%20a%20high.

M Global Services. (n.d.). What is Network Maintenance? [online] Available at: https://mglobalservices.com/knowledge-center/blog/what-is-network-maintenance/.

Velayudhan, D. (2023). 11 Critical Tasks in a Network Maintenance Checklist | ConnectWise. [online] www.connectwise.com. Available at: https://www.connectwise.com/blog/rmm/network-maintenance-checklist-tasks.